PLC 1 Lesson 7 Lab 1 – Comparisons

Program Name:  PLC 1 L7-1

Create a copy of the previous lab “PLC1 L6-2” using the save as and renaming it to “PLC 1 L7-1”.

Programming Guidelines:

Design a RSLogix 500 ladder logic program that will perform the functions described for each exercise.  Adhere to the following guidelines:

- Use the provided/simulated pushbuttons, lamps, and other available I/O.

- Plan your input and output assignments before you start programming.

- RUNG TITLES: Place only one rung title on the top rung of your program.  Make sure it is assigned to the rung and not the output.  Place the name of the exercise here (e.g.: Logic Exercise 3, Motor Exercise 2, Counter Exercise 1, etc.).  Do not place titles on remaining rungs as this will cause the start of a new page when printing.

- RUNG COMMENTS: In rung comment area, place your name then hit enter and add any comments related to the project.  Add comments without titles to other rungs as needed to explain the intended function of that rung.

- INSTRUCTION SYMBOLS:  Utilize instruction symbols that are given.  If there are none noted, please create your own.  [E.g.:  SW3, LP2, LS7, CR4, etc.]

- INSTRUCTION DESCRIPTIONS: Insert instruction descriptions whenever possible.
[E.g.: “capacitive sensor”, Conveyor Limit Switch, Motor Control Relay, Master Control Relay, etc.]

- Save and verify your program before downloading!  Then download your program to the PLC trainer and test to see that it works as intended.

Functional Specification:

- Modify the Parking lot light program from Lesson 6 to include the following changes and new features

- Add some indicators to help the attendant monitor lot condition.

- When the lot has 0 to 10 cars parked in it, turn on a light that indicates a nearly empty condition.

- When the lot has less than or equal to 20 cars, but more than 10 turn on an almost full indicator.

- The almost full indicator light should blink when the car count is greater than 20 cars.

- Also add a counter error indicator that lights if the lot car count exceeds 25 cars or is less than 0 cars.

Type of Device	Description				Address

Prox Sw			Car at Gate			I:0/0

Prox Sw			Car Exited Lot			I:0/1

PB NO			Lot Reset			I:0/2

CR Coil			Gate Up Relay		  	O:1/0

CR Coil			“No Vacancy” Sign		O:1/1

CR Coil			Lot Nearly Empty Light		O:1/2

CR Coil			Lot Almost Full			O:1/3

CR Coil			Car Count Error			O:1/4